MATERIAL SAFETY DATA SHEET OF POLYALUMINIUM CHLORIDE  (MSDS OF PAC)

1.  CHEMICAL NAME

Name On Label:            Polyaluminium Chloride

Alternative Name:           PAC

Formula:                  Al2Cl(OH)5

[Molecular Weight]:         174.45

2.  COMPOSITION AND INFORMATION ON COMPONENTS

Major Component:          Al2Cl(OH)5

Content:                   99%

Others Impurity:             1%

CAS No.:                  1327-41-9

3.  HAZARDS IDENTIFICATION

Designation:            unharmful

Application:            Polyaluminium Chloride( PAC ) is an inorganic high

polymer flocculating agent. Mainly used to purify

drinking water, for removing the iron cadmium, fluoride,

radioactive contamination, oil slicks etc in the special

water quality. Also used for industrial waste water

treatment, such as printing and dyeing water, in the

foundry, paper, pharmaceutical, leather, etc.

4.  FIRST AID MEASURES

Skin Contact:      Remove contaminated clothing. Wash affected area with

flowing water for at least 15 minutes. Rinse thoroughly. If

unusual symptoms are observed, seek medical advice.

Eye Contact:      Flush with copious amounts of water for at least 15 minutes. If

symptoms arise, seek medical advice.

Inhalation:        Remove to fresh air. Seek medical advice if symptoms persist.

Ingestion:         Rinse out mouth and drink some milk or egg white. In case of

unusual symptoms, seek medical attention.

5.  FIRE FIGHTING MEASURES

Extinguishing Medium Use fire fighting measures which suit the environment and take into account other materials which may be involved.

In general, water-based extinguishers should not be used for fires involving organic materials. Use carbon dioxide or dry powder. Protective Equipment Wear self-contained breathing apparatus and protective clothing.

6.  ACCIDENTAL RELEASE MEASURES

Personal Protection:   Isolation leakage polluted area, restricting access. Wear

protective equipment including dust mask and acid and alkali

resistant clothes. Do not direct contact with leakage.

A small leak:    Avoid dust and collect in sealed containers with clean shovel.

Large Leak:  Cover with plastic & canvas. Removed under the guidance of experts.

7.  STORAGE AND TRANSPORTATION

Storage:      Stored in a cool, dry, well ventilated warehouse, away from fire and

heat relative humidity remaining at 75% below. Packaging must be

sealed and not damp. Should not be stored with flammable, alkali,

alcohol, etc. and avoid mixing reservoir. Not survive long to avoid

deterioration. Storage areas should be equipped with suitable

containers to collect leakage.

Transportation:    Transport should strictly follow the provisions of International

Maritime Dangerous Goods Code.

8.  EXPOSURE CONTROLS AND PERSONAL PROTECTION

Respiratory:     Possible exposure to dust, should wear self-absorption filter

respirators; emergency rescue or evacuation, it is recommended to

wear breathing apparatus.

Eye:            Avoid eye contact. Wear safety spectacles or goggles.

Hands & Body:   Avoid skin contact. Wear rubber, acid and alkali resistant gloves

and protective clothing.

9.  PHYSICAL AND CHEMICAL PROPERTIES

Appearance:               Light yellow solid powder

Physical Constants:          m.p. 181-183℃

Molecular formula:          Al2Cl(OH)5

Formula Wt.:               185.22

Density:                   N/A

Fusing Point:               190(253kPa)

Boiling Point:               N/A

Relative Density (water= 1):    2.44

Relative Density (air= 1):      N/A

Saturated Vapor Pressure:      0.13(100℃)

Solubility:                  Soluble in water, alcohol, chloroform, carbon

tetrachloride, slightly soluble in benzene.

10.  STABILITY AND REACTIVITY

Stability:                  Stable

Specific Hazard:            N/A

Incompatibilities:           Flammable or combustible, alkaline, alcoholic

materials & water

11.  TOXICOLOGICAL INFORMATION

Acute Toxicity:               LD50: 3730 mg / kg (rat oral)

Irritating:	                 N/A

Subacute & Chronic Toxicity:         N/A

Reproductive Toxicity:               N/A

Carcinogenic:                      N/A

Mutagenicity:                      N/A

12.  ECOLOGICAL EFFECTS

General:           Take care to prevent chemicals from entering the ground, water

courses or drainage systems.

13.  DISPOSAL CONSIDERATIONS

Recover the product if possible. Operate according to local regulations.

- TRANSPORT INFORMATION

Road (RID/ADR):             not hazardous.

Rail (RID):                   not hazardous.

Air (ICAO/IATA):             not hazardous.

MARINE(IMO/IMDG):        not hazardous.

15.  REGULATORY INFORMATION

GB15892-2009, Water Treatment Chemical--Polyaluminium Chloride

- 16.  ADDITIONAL INFORMATION

The information contained herein is offered only as a guide to the handling of this specific material and has been prepared in good faith by technically knowledgeable personnel. It is not intended to be all-inclusive and the manner and conditions of use and handling may involve other and additional considerations. No warranty of any kind is given or implied and our company will not be liable for any damages, losses, injuries or consequential damages which may result from the use of or reliance on any information contained herein. This Material Safety Data Sheet is valid for three years.